| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 180 | 180 | 180 | 180 |

| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 0 | 180 | 180 | 180 |
| Wet | 0 |  |  |  |

Vacuum: test Low shaky.

Exhaust Valve: Stuck open, Burnt, Broken, Bent etc…

Piston: Hole in piston, Missing, Melted, etc…

Cylinder wall: Large Hole in wall.

Vacuum: test Low and steadily pulsating

Intake valve: Stuck open, burnt, broken, bent etc…

| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 50 | 50 | 50 | 50 |
| Wet | 90 | 90 | 90 | 90 |

Vacuum: Test Low and steady

Piston rings/Cylinder wall: Worn out beyond service lifetime

| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 45 | 50 | 45 | 55 |
| Wet | 45 | 50 | 45 | 55 |

Vacuum: Test low and steady possibly not testable due to, too low of compression.

Cam/Valve timing: Incorrect.

Timing chain: Worn timing gear, tensioner, broken gear

Timing belt: broken, missing cogs, failed tensioner.

Gears: broken damaged

Camshaft: Broken, worn camshaft lobes

| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 180 | 60 | 75 | 180 |
| Wet |  | 60 | 75 |  |

Due to cylinders 2 and 3 being next to each other their vicinity the cylinder head gasket is most likely cause.

| Cylinder | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Dry | 180 | 25 | 185 | 175 |
| Wet |  | 25 |  |  |

Vacuum: test Low shaky.

Exhaust Valve: Stuck open, Burnt, Broken, Bent etc…

Piston: Hole in piston, Missing, Melted, etc…

Vacuum: test Low and steadily pulsating

Intake valve: Stuck open, burnt, broken, bent etc…